HealthNest

Employee Wellbeing Monitoring System

ML Developer Guide v2 — WESAD (Wrist) to Live Prediction

What changed from v1 This version fixes 6 bugs in the original guide and rescopes the model for FYP-1. The biggest change: train on WESAD's WRIST (Empatica E4) signals, not the CHEST (RespiBAN) signals — your MAX30102 is a wrist PPG sensor, and the chest device has no BVP/PPG signal at all, so the old code would crash. See the change log below for everything else.

# 0. Change Log (v1 → v2)

| # | Problem in old guide | Fix in this version |
| --- | --- | --- |
| 1 | Guide reads data['signal']['chest']['BVP'] — this field doesn't exist. Chest has ECG, not PPG. | Read from data['signal']['wrist']['BVP'] instead — this is PPG, same sensing type as MAX30102. |
| 2 | 4 output classes (Healthy/Stressed/Fatigued/Overexerted), but WESAD only has ground truth for baseline and stress. Fatigued/Overexerted were invented by splitting the 'amusement' label with an SMA threshold. | FYP-1 model outputs 2 classes: Calm and Stressed. 4-class expansion is FYP-2 work, once you have your own collected data for fatigue/overexertion. |
| 3 | Labels (0/1/2/3) were downsampled using scipy resample(), which is an FFT-based interpolation. Applied to integer category labels, it invents fractional/garbage values at transitions. | Downsample labels by plain index picking (nearest-sample selection), never by FFT resampling. |
| 4 | Random train_test_split() on overlapping 50%-step windows. Neighbouring windows share samples, so this leaks the same subject's data into both train and test and inflates accuracy. | Use Leave-One-Subject-Out (LOSO) cross-validation: train on 14 subjects, test on the 1 held out, repeat for all 15. |
| 5 | Live Flask code computes RMSSD from HR values arriving every ~2 seconds. That is not heart-rate variability — HRV needs beat-to-beat timing, not an already-averaged BPM number. | Buffer raw PPG (IR) samples at 100 Hz as streamed from the band, run peak detection to get real beat-to-beat intervals, then compute RMSSD the same way as in training. |
| 6 | SMA threshold (1.5) and accelerometer units were taken from the chest device without adjustment. | Empatica E4 wrist ACC is in 1/64 g units — convert to g before computing SMA, and re-tune the threshold using your own data, don't reuse 1.5 blindly. |

# 1. Project Overview (FYP-1 scope)

HealthNest monitors industrial worker wellbeing using a custom ESP32 wristband (MAX30102 + MPU6050, optional skin temperature sensor). Signals are streamed over BLE to a Flutter app, forwarded to a Flask backend, and fed into a trained ML model.

For FYP-1, the model outputs one of two states:

- Calm

- Stressed

SpO2 (blood oxygen) is checked separately with a rule-based threshold and does NOT go into the ML model.

FYP-2 roadmap (not built now) Expand to 4 classes (add Fatigued, Overexerted) once real band data is collected from volunteers, since WESAD alone cannot support those two classes honestly. Add per-worker baseline calibration.

# 2. WESAD Dataset

## 2.1 What is WESAD

WESAD (Wearable Stress and Affect Detection) is a public dataset from 15 subjects wearing two devices at once: RespiBAN (chest) and Empatica E4 (wrist). Each sample is labeled with a ground-truth condition: baseline (relaxed), stress, or amusement.

Download: https://ubicomp.ifi.lmu.de/pub/datasets/WESAD/

## 2.2 Signals to Extract — use WRIST (Empatica E4), not chest

| WESAD path | Signal | Rate | Maps to your hardware |
| --- | --- | --- | --- |
| signal > wrist > BVP | Blood Volume Pulse (PPG) | 64 Hz | HR + HRV (MAX30102) |
| signal > wrist > ACC | Accelerometer X,Y,Z | 32 Hz | Activity (MPU6050) |
| signal > wrist > TEMP | Skin Temperature | 4 Hz | Skin temp sensor |
| label | Ground-truth label | 700 Hz | Training target |

Note: the label array is always sampled at 700 Hz regardless of device — it comes from the study protocol timeline, not from a sensor.

## 2.3 Label Mapping (FYP-1: 2 classes)

| WESAD label | WESAD meaning | Your class | Your label |
| --- | --- | --- | --- |
| 0 | Transient / not a condition | Skip | — |
| 1 | Baseline (relaxed) | Calm | 0 |
| 2 | Stress (public speaking task) | Stressed | 1 |
| 3, 4+ | Amusement / other | Skip for FYP-1 | — |

Dropping amusement/other windows entirely for FYP-1, rather than relabeling them as Fatigued/Overexerted — there is no real evidence in WESAD for those two states.

# 3. Loading WESAD Data

Each subject has a separate .pkl file (S2.pkl ... S17.pkl, S12 is usually excluded per WESAD's own readme). Load each one like this:

import pickle import numpy as np with open('S2.pkl', 'rb') as f: data = pickle.load(f, encoding='latin1') # WRIST signals — matches our MAX30102/MPU6050 hardware bvp = data['signal']['wrist']['BVP'] # shape: (n, 1) at 64 Hz acc = data['signal']['wrist']['ACC'] # shape: (n, 3) at 32 Hz, units = 1/64 g temp = data['signal']['wrist']['TEMP'] # shape: (n, 1) at 4 Hz labels = data['label'] # shape: (n,) at 700 Hz subject_id = data['subject'] # e.g. 'S2' — needed for LOSO split

# 4. Resampling to a Common Rate

Target rate: 64 Hz (the BVP rate). ACC and TEMP are continuous signals, so FFT-based resample() is fine for them. Labels are categories, not continuous — resample() must NOT be used on them.

from scipy.signal import resample n_samples = len(bvp) # use BVP length as the target length # ACC (32 Hz -> 64 Hz) and TEMP (4 Hz -> 64 Hz): continuous signals, resample() is fine acc_resampled = resample(acc, n_samples) temp_resampled = resample(temp, n_samples) # Labels (700 Hz -> 64 Hz): CATEGORY data — pick nearest original sample per output # index, do not interpolate or FFT-resample integer class labels. orig_len = len(labels) idx = np.linspace(0, orig_len - 1, n_samples).round().astype(int) labels_resampled = labels[idx]

# 5. Sliding Window

At 64 Hz, 60 seconds = 3840 samples. 50% overlap (30 second step):

window_size = 64 * 60 # 3840 samples = 60 seconds step_size = 64 * 30 # 1920 samples = 30 seconds (50% overlap) for start in range(0, n_samples - window_size, step_size): end = start + window_size bvp_window = bvp[start:end].flatten() acc_window = acc_resampled[start:end] temp_window = temp_resampled[start:end].flatten() label_window = labels_resampled[start:end] valid = label_window[label_window > 0] if len(valid) == 0: continue window_label = np.bincount(valid).argmax() # skip mixed windows (more than one condition inside it) if len(np.unique(label_window[label_window > 0])) > 1: continue # skip windows outside our FYP-1 classes (keep only label 1 and 2) if window_label not in (1, 2): continue

# 6. Feature Engineering (inside each window)

## 6.1 HR_mean — Heart Rate

from scipy.signal import find_peaks peaks, _ = find_peaks(bvp_window, distance=40) # min 40 samples apart at 64 Hz rr_intervals = np.diff(peaks) / 64.0 # seconds HR_mean = 60.0 / np.mean(rr_intervals)

## 6.2 RMSSD — Heart Rate Variability (key stress feature)

Low RMSSD = stressed. High RMSSD = relaxed.

rr_diff = np.diff(rr_intervals) RMSSD = np.sqrt(np.mean(rr_diff ** 2))

## 6.3 SMA — Signal Magnitude Area (activity level)

Convert E4 wrist ACC from 1/64 g units to g before computing SMA:

acc_g = acc_window / 64.0 # E4 wrist accelerometer -> g units ax, ay, az = acc_g[:, 0], acc_g[:, 1], acc_g[:, 2] SMA = np.mean(np.abs(ax) + np.abs(ay) + np.abs(az))

## 6.4 TEMP_mean and TEMP_delta

TEMP_mean = np.mean(temp_window) # TEMP_baseline = mean skin temp from this subject's label==1 (calm) windows, # computed once per subject before this loop. TEMP_delta = TEMP_mean - TEMP_baseline

# 7. Final Label Assignment

if window_label == 1: final_label = 0 # Calm elif window_label == 2: final_label = 1 # Stressed # window_label is already filtered to (1, 2) in the windowing step above

# 8. Training the Model — Leave-One-Subject-Out

A random train_test_split leaks data because overlapping windows from the same subject end up in both sets. Evaluate per-subject instead:

import numpy as np from sklearn.ensemble import RandomForestClassifier from sklearn.model_selection import LeaveOneGroupOut from sklearn.metrics import classification_report import pickle # X = feature matrix, y = labels, groups = subject id per row (e.g. 'S2', 'S3', ...) # features: [HR_mean, RMSSD, SMA, TEMP_mean, TEMP_delta] X = np.array(features_list) y = np.array(labels_list) groups = np.array(subject_list) logo = LeaveOneGroupOut() all_true, all_pred = [], [] for train_idx, test_idx in logo.split(X, y, groups): model = RandomForestClassifier(n_estimators=100, random_state=42) model.fit(X[train_idx], y[train_idx]) preds = model.predict(X[test_idx]) all_true.extend(y[test_idx]) all_pred.extend(preds) print(classification_report(all_true, all_pred, target_names=['Calm', 'Stressed'])) # Train the FINAL model on ALL subjects for deployment final_model = RandomForestClassifier(n_estimators=100, random_state=42) final_model.fit(X, y) with open('healthnest_model.pkl', 'wb') as f: pickle.dump(final_model, f)

Report the LOSO score, not a random-split score The classification_report from the LOSO loop is the number to put in your FYP report — it estimates how the model performs on a person it has never seen, which is the real deployment scenario. A random-split score will look better but is misleading.

# 9. Flask Backend — Live Prediction

The band streams raw PPG (IR) samples at 100 Hz and raw accelerometer samples at 50 Hz per the BLE packet spec (not a pre-computed BPM value every 2 seconds). Buffer 60 seconds of raw samples per worker, then extract the SAME features used in training.

## 9.1 Incoming raw samples (from PPG and Motion BLE characteristics)

# Per worker, maintain rolling buffers: ppg_buffer = [] # raw IR samples, ~100 Hz -> 6000 samples for 60 sec acc_buffer = [] # (ax, ay, az) in g, ~50 Hz -> 3000 samples for 60 sec temp_buffer = [] # skin temp readings, ~1 Hz -> 60 samples for 60 sec

## 9.2 Feature Extraction and Prediction

import pickle import numpy as np from scipy.signal import find_peaks with open('healthnest_model.pkl', 'rb') as f: model = pickle.load(f) PPG_RATE = 100 # Hz, matches the band's PPG sample rate def predict_state(ppg_buffer, acc_buffer, temp_buffer, worker_baseline_temp): ppg = np.array(ppg_buffer) # Same peak-detection approach as training, adjusted for the band's rate min_distance = int(0.4 * PPG_RATE) # ~40 samples at 100 Hz peaks, _ = find_peaks(ppg, distance=min_distance) rr_intervals = np.diff(peaks) / PPG_RATE HR_mean = 60.0 / np.mean(rr_intervals) rr_diff = np.diff(rr_intervals) RMSSD = np.sqrt(np.mean(rr_diff ** 2)) acc = np.array(acc_buffer) # already in g from the app/backend parsing layer SMA = np.mean(np.abs(acc[:, 0]) + np.abs(acc[:, 1]) + np.abs(acc[:, 2])) TEMP_mean = np.mean(temp_buffer) TEMP_delta = TEMP_mean - worker_baseline_temp X_live = [[HR_mean, RMSSD, SMA, TEMP_mean, TEMP_delta]] prediction = model.predict(X_live)[0] states = {0: 'Calm', 1: 'Stressed'} return states[prediction]

worker_baseline_temp: capture the worker's first ~5 minutes of TEMP_mean at shift start and store it as their baseline for that shift.

## 9.3 SpO2 Check (separate from the model, unchanged)

def check_spo2(spo2_value, worker_id): if spo2_value < 95: trigger_alert(worker_id, f'Low SpO2 detected: {spo2_value}%')

# 10. Quick Reference Summary

| Step | What to do |
| --- | --- |
| 1 | Load each subject's .pkl file (S2 to S17) |
| 2 | Extract BVP, ACC, TEMP, labels from the WRIST device only |
| 3 | Resample ACC/TEMP to 64 Hz with resample(); downsample labels by index picking, not resample() |
| 4 | Apply 60 sec window, 30 sec step; keep only windows labeled 1 (calm) or 2 (stress) |
| 5 | Compute 5 features per window: HR_mean, RMSSD, SMA (converted to g), TEMP_mean, TEMP_delta |
| 6 | Map to 2 classes: Calm (0), Stressed (1) |
| 7 | Train Random Forest, evaluate with Leave-One-Subject-Out, then fit a final model on all subjects |
| 8 | Save model as healthnest_model.pkl |
| 9 | Flask buffers 60 sec of RAW PPG + accel samples per worker, extracts the same 5 features, predicts |
| 10 | SpO2 stays a separate threshold check — never passed to the model |

# 11. Feature Summary Table

| Feature | Source | How computed | What it indicates |
| --- | --- | --- | --- |
| HR_mean | PPG (MAX30102) | Peak detection on PPG -> RR intervals -> 60/mean(RR) | Overall cardiac load |
| RMSSD | PPG (MAX30102) | sqrt(mean(diff(RR)^2)) | Stress: low RMSSD = stressed |
| SMA | ACC (MPU6050) | mean(¦ax¦+¦ay¦+¦az¦), in g | Physical activity level |
| TEMP_mean | Skin temp sensor | mean of 60 sec window | Skin temperature |
| TEMP_delta | Skin temp sensor | TEMP_mean − subject/worker baseline | Temperature rise from rest |

HealthNest — ML Guide v2